# Lane Keep Assist (LKA) via Classical Computer Vision

# Abstract

I implemented a Path-A classical LKA: crop a road ROI, warp to a bird’s-eye view (IPM/BEV), threshold lane paint using brightness (LAB-L with CLAHE) and image gradients, extract lane pixels with a bottom histogram + sliding windows, fit second-order polynomials for left/right lanes, and smooth coefficients over time. It produces an annotated MP4 and a per-frame CSV.

# Method

## Pipeline

- ROI: Crop the lower half (road).

- IPM/BEV: Warp ROI to top-down with 4 source points normalized to the ROI; destination is a fixed rectangle.

- BEV thresholding:

  - Convert BEV color to LAB and take L* (lightness). Apply CLAHE (clip≈2.0; grid≈8) and mild bilateral filtering -> Lf.

  - White paint gate: adaptive threshold on L* from the center corridor percentile (e.g., 90th − 8), with a small back-off if too sparse.

  - Gradient gate: threshold the gradient magnitude |∇L| (Sobel) with mag_min (currently 20).

  - Combine: mask = white AND grad.

  - Specular cleanup: remove tiny bright V-blobs (HSV-V).

  - Anisotropic morphology: horizontal open (e.g., 7×1) then vertical close (e.g., 3×21).

  - Tall/thin filter: keep connected components with lane-like geometry (h ≥ 50, w ≤ 54, area ≥ 160).

- Lane finding: Bottom histogram (left/right bands), sliding windows (n≈9, margin≈120, minpix≈30), fit 2nd-order x=f(y) polynomials.

- Temporal smoothing: EMA on coefficients (α≈0.88), “keep-last” when current confidence is weak.

- Confidence/flags: combine pixel count, fit residual, and Δcoeff vs. t−1 → conf∈[0,1]. detected = (conf > τ) with τ=0.6.

- Overlay & CSV: Project fits back, left=green, right=blue; dashed gray when “Not detected”; HUD with YES/NO + confidences. CSV columns: frame_id,left_detected,right_detected,left_conf,right_conf,lat_offset_m

## Calibration & ROI

- ROI: Chosen to exclude hood/dashboard while keeping enough look-ahead.

- IPM source polygon: Four ROI-normalized points laid over the ego-lane corridor (validated with a first-frame screenshot).

- Corridor band-pass (bandpass_x ≈ [0.40, 0.60]) is used for percentile estimation; applying it to the final mask further reduces non-lane clutter

## Thresholds

- White: auto L* from corridor 90th percentile − 8; fallback l_white_min=178.

- Gradient: grad_mag_min=20 on |∇L|.

- Morphology/shape: open 7×1, close 3×21, keep tall/thin CCs.

- Temporal: EMA α=0.88, min_conf_for_update=0.35, keep_last_on_drop=true

# Metrics

## Video 1 (n=280)

- Detection: Left 0.889, Right 0.711; mean conf L 0.848, R 0.711.

- Median detected-run length: L 91 frames, R 15. Dropouts per 1k: L 7.1, R 39.3.

- Stability: offset mean -0.149 m (slightly left of center), std 0.075 m; median |Δ| 0.001 m, p95 0.040 m.

- Interpretation: Strong baseline performance (tunnel/night). Left rail very stable, right rail intermittent but acceptable. Offset is steady and smooth.

## Video 2 (n=1199)

- Detection: Left 0.253, Right 0.218; mean conf L 0.330, R 0.311.

- Median runs: L 5, R 4. Dropouts per 1k: 39.2 (both).

- Stability: offset mean -0.090 m, std 0.463 m; median |Δ| 0.000 m, p95 0.074 m.

- Interpretation: Daytime/highway conditions with overexposure/low contrast. Low recall on both sides; the large offset std reflects inconsistent lane capture or side switching; per-frame jitter is modest (p95 Δ≈7 cm), so drift is slow but often wrong.

## Video 3 (n=1260)

- Detection: Left 0.299 (overstated), Right 0.412; mean conf L 0.443, R 0.519.

- Median runs: L 2, R 4. Dropouts per 1k: L 93.7, R 88.9.

- Stability: offset mean +0.493 m, std 0.289 m; median |Δ| 0.000 m, p95 0.046 m.

- Interpretation: The left rail frequently latched onto a non-lane structure in early frames (false positive), inflating the apparent left detection rate. Right rail is slightly better but still fragile. High dropout rates confirm instability.

## Video 4 (n=251)

- Detection: Left 0.398, Right 0.510; mean conf L 0.462, R 0.584.

- Median runs: L 5, R 5. Dropouts per 1k: L 51.8, R 95.6.

- Stability: offset mean +0.105 m, std 0.136 m; median |Δ| 0.002 m, p95 0.057 m.

- Interpretation: Mixed daylight scene; right rail modestly better than left. Offset variability moderate, frequent short dropouts, especially on the right.

## Summary

- Best case: Video 1 (night/tunnel) - LAB-L + gradients is well suited for bright white paint against a dark road; left side is particularly strong.

- Challenging: Videos 2–4 (daytime highways) - overexposed asphalt reduces L-contrast; gradient magnitude admits more non-vertical edges, increasing false positives and dropouts; Video 3 left shows an extended early false positive.

# Discussion

ODD (Operational Design Domain): paved multilane highways/urban arterials with painted lane lines; dry/light rain; daylight and tunnels; moderate curvature; no heavy construction/ghost markings.

Observed failure modes -> LKA behavior

- Overexposure / low contrast (Videos 2–4)
Symptom: paint vs. asphalt not separable; frequent dropouts and side confusion.
LKA action: Warn (visual/audible) when either side falls below τ; reduce assist torque; disengage if both sides < τ for >0.5 s.

- False positive on structural edges (Video 3 left, early)
Symptom: latch onto barrier/vehicle edge.
Mitigation: require vertical-edge dominance (Sobel-X) + corridor band-pass; track curvature/offset continuity; down-weight sudden lateral jumps.
LKA action: Warn and suppress torque until the track stabilizes for K frames.

- Intermittent right rail (Video 1)
Symptom: shorter runs on right.
Mitigation: stronger temporal prior (narrow track_margin), histogram bands tuned to the scene; hold-last for brief occlusions.
LKA action: Allow gentle centering if one rail remains solid; warn on long single-side loss.

- Merges/exits/curves
Symptom: abrupt polynomial shifts; window tracker jumps rails.
Mitigation: curvature sanity checks; 3rd-order fit allowed but bounded; decay updates during topology changes.
LKA action: Cap torque, warn; disengage if both rails uncertain.